CCM1100S Diagnostic Tool – End‑to‑End Technical Project Guide

1. Overview

This project implements a wireless diagnostic tool for the CCM1100S ECU (Electronic Control Unit) used in commercial vehicles. A Raspberry Pi acts as a BLE‑to‑CAN bridge, allowing a Flutter‑based mobile app to read and write UDS (ISO 14229) parameters over Bluetooth.

Key capabilities:

- Read/write Data Identifiers (DIDs) via UDS service 0x22/0x2E

- Multi‑frame ISO‑TP (ISO 15765-2) handling for messages >7 bytes

- Security access (0x27) with AES‑ECB seed/key exchange

- Automatic ECU connection monitoring and status reporting

- Persistent operation as a systemd service on the Pi

2. Hardware Setup

Required Components

- Raspberry Pi (3B/4/5) with Raspbian Bookworm

- CAN transceiver (e.g., MCP2515) connected to SPI or USB‑CAN adapter

- CCM1100S ECU with CAN bus (250 kbit/s, extended IDs)

- Smartphone (Android) for Flutter app

CAN Interface Configuration

# Add to /etc/rc.local or create a systemd service ip link set can1 down ip link set can1 up type can bitrate 250000

Verify with:

ip link show can1   # Should show state UP candump can1        # Monitor traffic

Bluetooth on Pi

sudo apt install bluez bluez-tools python3-dbus sudo systemctl enable bluetooth sudo systemctl start bluetooth

3. Raspberry Pi Bridge (Python)

The bridge is a Python 3.13 application that advertises a BLE GATT service, receives JSON commands from Flutter, translates them to UDS over CAN using ISO‑TP, and returns ECU responses as JSON notifications.

3.1 Project Structure

CCM1100S_Diagnostic/ 
├── main.py 
├── ble/ 
│   ├── ble_server.py 
│   └── command_handler.py 
├── core/ 
│   ├── can_bus.py 
│   ├── iso_tp.py 
│   ├── uds_client.py 
│   └── security_manager.py 
├── config/ 
│   └── settings.py 
└── utils/
    └── logger.py

3.2 CAN Bus Manager (can_bus.py)

Creates a SocketCAN socket with a hardware filter for the ECU’s response ID (0x1BDAF108). Handles the “No buffer space available” error gracefully when the ECU is off (keeps socket open). Provides send_message() and receive_message().

3.3 ISO‑TP Handler (iso_tp.py)

Implements ISO 15765-2 exactly as the working manual script (check.py):

- Single frame (≤7 bytes): PCI byte = length, padded to 8 bytes.

- Multi‑frame (>7 bytes): send First Frame, wait for Flow Control, parse STmin, send Consecutive Frames with timing, reassemble response.

3.4 UDS Client (uds_client.py)

Implements UDS services: diagnostic session control (0x10), read data by identifier (0x22), write data by identifier (0x2E), tester present (0x3E). Includes data scaling per ECU manual:

| DID | Description | Raw bytes | Scaled output |
| --- | --- | --- | --- |
| 0x220F | System Voltage | 0900 | 23.04V |
| 0x2210-0x2212 | Axle Angles | 0000 | 0.0° |
| 0x2213-0x2214 | Control % | 0000 | 0% |
| 0x2215-0x221C | Currents | 3636 | 13878mA |
| 0xF190 | VIN | 7765... | wewewe... |
| 0xF18C | Serial Number | 00000000 | 0 |

3.5 Security Manager (security_manager.py)

Uses AES‑ECB with fixed 16‑byte key (TCHRMVHA2BPX3ULC). Handles multi‑frame seed responses (up to 18 bytes) and detects “already unlocked” when seed is all zeros.

3.6 BLE Server (ble_server.py)

Built with dbus‑next and BlueZ. Service UUID: 12345678-1234-1234-1234-123456789ABC. Characteristic UUID: 87654321-4321-4321-4321-CBA987654321. Supports indicate (reliable) and write. Connection monitor pushes ECU_CONNECTED/ECU_DISCONNECTED status to Flutter.

3.7 Main Loop (main.py)

Initialises all components, starts BLE server even if ECU is offline, automatically restarts on any exception, handles SIGINT/SIGTERM for clean shutdown.

4. Flutter App (Mobile)

The Flutter app provides a technician‑friendly interface to send UDS commands and display responses.

4.1 Project Structure (simplified)

lib/ 
├── main.dart 
├── config/app_config.dart 
├── services/ble_service.dart 
├── controllers/uds_controller.dart 
├── models/uds_response.dart 
├── views/screens/uds_screen.dart 
└── utils/hex_formatter.dart

4.2 BLE Service (ble_service.dart)

Uses flutter_blue_plus. Connects to the Pi’s characteristic, enables indications, listens to lastValueStream, separates connection status messages from command responses.

4.3 UDS Controller (uds_controller.dart)

Sends JSON commands: {\"command\":\"read_did\",\"did\":\"0x220F\",\"id\":timestamp}. Provides readDid(), writeDid(), unlock(). Maintains isLoading state and 5‑second timeout.

4.4 UI Screens (uds_screen.dart)

Dropdown populated from a Map of DID names → hex values. Two operation modes: TEXT mode (user enters string, auto‑converted to hex) and HEX mode (user enters raw hex bytes). Displays response in a console‑like box with status, hex, and human‑readable value. “WRITE OKAY” LED pulses on successful write.

4.5 DID Length Validation

Each DID has a fixed byte length (from ECU manual). The app enforces this:

int getRequiredLength(String did) {   
  
  switch (did) {     
    case "F187": case "F188": case "F191": case "F1A0": 
       return 15;     
    case "F190": case "F1A1": 
       return 17;     
    case "F1A6": 
       return 20;     
    case "220D": case "220E": case "F18C": case "F192": 
       return 4;     
    default: 
       return 0;   
  } 
}

5. Communication Protocol (JSON over BLE)

5.1 Request from Flutter

{

"command": "read_did", 
  "did": "0x220F",
  "id": 1776238147593

}

5.2 Response from Pi (success)

{

"success": true
  "did": "0x220F",
  "data": "32332e303456",
  "value": "23.04V",
  "raw": "32332e303456",
  "id": 1776238239163

}

5.3 Connection Status (automatic)

{
  "type": "connection_status",
  "status": "ECU_CONNECTED",
  "success": true,
  "message": "ECU is online and responding"
}

6. Deployment on Raspberry Pi

6.1 Systemd Service (auto‑start on boot)

# /etc/systemd/system/uds-bridge.service 
[Unit]

Description=UDS BLE Bridge for CCM1100S ECU

After=network.target bluetooth.service can-setup.service

Wants=can-setup.service

[Service]

Type=simple

User=pi

WorkingDirectory=/home/pi/CCM1100S_Diagnostic

ExecStart=/home/pi/CCM1100S_Diagnostic/venv/bin/python /home/pi/CCM1100S_Diagnostic/main.py

Restart=always

RestartSec=5

StandardOutput=append:/var/log/uds-bridge.log

StandardError=append:/var/log/uds-bridge-error.log

[Install]

WantedBy=multi-user.target

6.2 CAN Setup Service

#/etc/systemd/system/can-setup.service

[Unit]

Description=CAN Bus Setup

Before=uds-bridge.service

[Service]

Type=oneshot

RemainAfterExit=yes

ExecStart=/bin/bash -c '/sbin/ip link set can1 down 2>/dev/null; /sbin/ip link set can1 up type can bitrate 250000'

[Install]

WantedBy=multi-user.target

6.3 Enable and Start

sudo systemctl daemon-reload

sudo systemctl enable can-setup.service uds-bridge.service

sudo systemctl start uds-bridge.service

6.4 View Logs

sudo journalctl -u uds-bridge.service -f tail -f /var/log/uds-bridge.log

7. Troubleshooting Common Issues

| Symptom | Likely Cause | Solution |
| --- | --- | --- |
| Bridge not advertising | BlueZ not running or dbus‑next missing | sudo systemctl restart bluetooth; install dbus‑next |
| CAN buffer space errors | ECU offline or CAN termination missing | Ignored by bridge; check physical connection |
| Security access fails with NRC 0x7F | ECU not in Extended session | Bridge automatically retries with Tester Present |
| Flutter receives only echo | Notifications not enabled | Ensure setNotifyValue(true) is called |
| Write fails with NRC 0x13 | Wrong data length for DID | Use the length‑validation function in the app |
| Multi‑frame timeout | STmin not respected | ISO-TP handler now uses STmin from Flow Control |
| Service fails to start | Missing #!/bin/bash in rc.local | Use systemd CAN service instead |

8. Conclusion

The CCM1100S Diagnostic Tool is a complete, production‑ready solution for wirelessly diagnosing and programming the vehicle’s ECU. The Raspberry Pi bridge is robust (auto‑restart, connection monitoring, multi‑frame ISO‑TP) and the Flutter app provides an intuitive interface for technicians. All critical UDS services (read, write, security access) are implemented and tested.

Future enhancements could include: DTC reading/clearing (service 0x19/0x14), live data streaming with periodic polling, session logging and export, and over‑the‑air (OTA) firmware updates via UDS.